Sampling Protocol (2/12/2013): Stable Isotopes of water (18O & 2H)

Collection type: 60 mL RUS

USGS Schedule/Lab Code: SCH 1142

Analyzing laboratory: Reston Stable Isotope Laboratory

Bottle Type: 1 60-mL glass bottle, cone cap

One Stop Order Info: Q411FLD

Filtered/Unfiltered: Unfiltered

Treatment: No Preservative

Additional bottle instructions: None

Sampling instructions:

- Top fill with raw, unfiltered water, 3/4 full

- Fill bottle ¾ to leave some headspace in case the  bottle freezes. Cap tightly and tape cap with electrical tape (wind tape clockwise to prevent cap from unscrewing)

- The RSIL will use QWDX to enter sample information and data into NWIS.

Questions regarding sampling and shipping procedures can be directed to: Bryant Jurgens (916) 278-3275 (bjurgens@usgs.gov)

Shipping Protocol (2/25/2013): Stable Isotopes of water (18O & 2H)

SCH 1142 Stable Isotopes of Water – Reston Stable Isotope Laboratory

- Arrange samples in order according to date/ time.

- Log sample information (Site ID, Date, Time, etc.) into RSIL Sample Submission Form –spreadsheet.

http://isotopes.usgs.gov/lab/services.html

- Be sure to fill out submittal form as completely as possible. Code descriptions are included on separate tabs.

- Update study unit account #.  Avoid logging samples from different study units on the same sheet.

- Note any comments pertaining to samples in spreadsheet.

- Make sure all caps are taped.

- Pack samples with foam/bubble sleeves securely into a box within a box and double bagged in case glass breaks. Put packing material between boxes for cushioning.

- Included a printed copy of the Sample Submission Form in the box.

- Highlight the list of samples contained in the box if using one excel sheet for many boxes.

- Separate excel sheets by batch # according to shipment (e.g. three boxes of samples shipped on a particular day would be included in batch # 1. The next shipment of samples would be batch #2.)

- Email Excel submittal form to: isotopes@usgs.gov and tlarnold@usgs.gov corresponding to the samples being shipped.

- Samples can be shipped at the end of the week (Thursday) and shipped FedEx overnight to:

USGS, Reston Stable Isotope Lab

431 National Center

12201 Sunrise Valley Dr.

Reston VA, 20192

703-648-5859

DO NOT SHIP TO ARRIVE ON SATURDAY

Contacts for questions –

Hiaping Qi, isotopes@usgs.gov

– Bryant Jurgens, (916) 278-3275