_______________________________________________________________________________

INPUT AND OUTPUT FILE SPECIFICATION

The names of the input and output files are entered interactively.  The file

names may contain a full path name, a local name, or a relative name.  Maximum

length of a file name is 64 characters.

_______________________________________________________________________________

DATA INPUT SPECIFICATION

RCRD  COLUMNS  FORMAT  VARIABLE  DESCRIPTION

----  -------  ------  --------  ---------------------------------------------

1     1-28   A28     XIDENT    problem identification

29-32  F4.0    Q         For METD = 1, blank

For METD = 2, discharge for downstream

station.  In cfs or cms

depending on value of IUNITS.

For METD = 3, blank

33-37  F5.0    BP        Average barometric pressure for study date.

Should not be corrected to an equivalent

sea level reading.  Program assumes that

any value greater than 50 is in millimeters,

values less than 50 are assumed to be inches.

38-39  I2      IPRNT     Indicator flag for interpolated values

0 - don't print out

1 - print out

40-42  I3      METD      Method of analysis

1 - single station (stream)

2 - upstream - downstream

3 - lake or reservoir

43-47  F5.2    TT        For METD = 1, leave blank

METD = 2, travel time between stations,

in decimal to 0.25 hour.

METD = 3, leave blank

include the following 3 for lake or reservoir (METD = 3)

48-52  I5      NN        Number of horizontal slices in lake

Maximum of 10.

53-58  F6.0    DELTZ     Depth interval for lake segments (thickness

of lake slice), in meters.  Interval must

remain constant from the surface to the

bottom for an estimate of community

metabolism, or from the surface to the

lower limit of the euphotic zone if only

phytoplankton metabolism is of interest.

59-64  F6.0    DIFCL     Atmospheric diffusion constant at surface,

(g o(2) / m**3) / hr

2      1-3   F3.1    DEPTH     Depth of water for stream analysis, in meters.

for METD = 1, depth at sample point

METD = 2, average depth

METD = 3, leave blank

8-11  2I2     IDAY      Time of sunrise to nearest quarter hour,

in military time.  Obtain from local weather

bureau or calculate using an ephemeris.

12-15  2I2     INIGHT    Time of sunset to nearest quarter hour,

in military time.  Obtain from local weather

bureau or calculate using an ephemeris.

16-18  I3      ---       The plot option is not available with this

version of the program.

19-21  A3      TEST      Indicator flag for conductivity data

YES - conductivity data entered on record 4

blank - salinity data entered on record 4

22-23  I2      N20       Indicator flag for conductivity/salinity

correction.

1 - conductivity corrected to 25 degrees C

0 - conductivity data is uncorrected

0 - salinity data is entered

0 - data are for freshwater

24-29  F6.0    AREA      For METD = 1, leave blank

METD = 2, area of reach contained

between upstreeam and down-

stream stations, in meters

squared or ???? depending on

value of IUNITS

METD = 3, leave blank

30-35  F6.0    VEL       For DIFC = 99, Average velocity of stream, in

feet/second or meters/second,

depending on value of IUNITS.

36-40  I5      IUNITS    Indicator flag for units of measure

0 - input is in English units

1 - input is in metric units

3     1-2   I2      NOBS      Number of observations for each station,

each observation corresponding to a time.

For METD = 3, this represents the number of

observations for each lake slice at that

station.  Maximum of 97.

9-16  F8.0    DIFC      For METD = 1 or 2, diffusion coefficient for

streams, in (g o(2)/m**3) / hr.

If 99 entered, a constant will

be calculated.

For METD = 3, the internal dispersion

coefficient for each slice, in

square cm / second.

17-24  F8.0     SALIN    Salinity for marine waters, in g/l.

Entered if constant.  If a non-zero

value is entered, it will be used instead

of any values entered on record type 4.

25-32  F8.0     K2       User supplied value for K2.  Used in the

computation of the diffusion coefficient.

If zero or blank, program computes value

using original formulation for k

NOBS record type 4's

4      1-4  2I2       ITM      Time of observation, in military time to

the nearest quarter hour.

9-16  F8.0     Temp     Temperature (degrees Celsius)

17-24  F8.0     DisO     Disolved oxygen (mg/l)

25-32  F8.0     Salcon   For TEST = "   ", Salinity in g/l

TEST = "YES"  conductivity

in mu mhos / cm**2 * 10**4