mfgui.doc                                                August 12, 1998

This file documents the new features added in version 1.4 of MODFLOW-GUI,

which features remain in the latest version of MODFLOW-GUI.

Data files are exported while in the MODFLOW FD Grid layer by selecting

Export from the File Menu, and then selecting Run MODFLOW.  In version

1.4, a new dialog box appears in which you can specify the data files to

be exported by selecting check boxes.  Those check boxes whose label does

not exactly correspond to a single data file to be exported are listed

below with the data files to which they correspond.

Check box                      Corresponding data file(s)

---------                      --------------------------

Basic                          Misc. Controls

Initial hydraulic head

Prescribed head

Stress period

Output Control                 Output Files

Output Controls

Block-Centered Flow            Formation hydraulic properties

Matrix Solution Method         Data file for the DE4, SIP, SOR or PCG

matrix solution method

The data files exported can thus be limited to those affected by changes

made to the geospatial information layers and the Project Information.

However, it is important to note that if there are any changes in the

MODFLOW FD Grid layer, or if the number of geologic units has been

changed, or if the vertical discretization within geologic units has been

changed, all data files should be re-exported.  If you are not an

experienced user of MODFLOW-96 and have a question regarding which data

files should be exported, it is best to export all data files to be

safe.

Examples of cases where only single data files need to be exported:

- You wish to change the matrix solution method, but all geospatial

information has remained the same.  Only the box for the Matrix

Solution Method should be checked.

- From a previous simulation, you have only changed pumping rates at

wells or the top and bottom elevations of the pumped intervals of

wells in the layers Wells Unit[i].  Only the data file for well

information needs to be exported.

- From a previous simulation, you have only changed the spatial

distribution of hydraulic properties (hydraulic conductivity,

specific storage, etc.) but have not made changes in the MODFLOW FD

Grid layer.  Only the Block-Centered Flow data file needs to be

exported.

In version 1.4, selected geospatial information is checked upon

exporting.  Errors or warnings detected during the export are written to

a file named Rootname.ERR, where Rootname is the file name specified in

the Output Files dialog box.

If the elevation of the bottom of a Line or Area River in a geologic unit

does not lie between the top and bottom elevation of the geologic unit,

an error message is written to the Rootname.ERR file.  This error message

will only be written once during the export, even if there are many grid

cells where this condition occurs; the error message is a signal to check

all information in the Line or Area River geospatial information layers.

For grid cells where this condition occurs, the information associated

with the Line or Area River is placed in the top model layer of the

geologic unit under consideration and the bottom elevation and other

geospatial information associated with the Line or Area River is exported

to the Rootname.RIV file unaltered.

The elevation of Line and Area Drains in a geologic unit is checked and

error messages are written to the Rootname.ERR file as described above.

For grid cells where the elevation of the Line or Area Drain does not lie

between the top and bottom elevation of the geologic unit, the

information associated with the Line or Area Drain is placed in the top

model layer of the geologic unit under consideration and the elevation

and other geospatial information associated with the Line or Area Drain

is exported to the Rootname.DRN file unaltered.