# USGS PROJECT PROPOSAL COVER SHEET

# Central Region

| Center: Iowa Water Science Center |  |  |  | Proposal No. IA-09A |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Program Title: Streamest, A new approach for estimating stream flow and water quality |  |  |  | Initial date: 6-1-07 |  |  |  |
|  |  |  |  | Revised date (if applicable): 10-1-08 |  |  |  |
| Short Title: Streamest |  |  |  | Program Number: 8591D7E |  |  |  |
|  |  |  |  | Begin date (mo&yr): 10-1-08 |  |  |  |
| Program Chief: Daniel Christiansen, Jon Nania, |  |  |  | End date (mo&yr): 9-30-13  |  |  |  |
|  |  |  |  | Type: Research Resource Appraisal Data |  |  |  |
| Source of Funds: | Fed | Coop  | OFA | Funding/Expenditures by Fiscal Year: |  |  |  |
| Type of Funds: | C | F | R |  | Total | Direct | Unmatched |
| Cooperating Agency: Iowa Department of Natural Resources Customer Number: IA004 Funding Status:  High Low Planned |  |  |  | 2009 | $995,000 | $0 | $99,500 |
|  |  |  |  | 2010 | $1,006,000 | $0 | $100,600 |
|  |  |  |  | 2011 | $949,000 | $0 | $95,000 |
|  |  |  |  | 2012 | $1,052,000 | $0 | $105,300 |
|  |  |  |  | 2013 | $975,000 | $0 | $97,500 |
|  |  |  |  | Total | $4,977,000 | $0 | $497,900 |
| Attach to program proposal, which should be written in the format described in Central Region Memorandum 2004-01. (http://wwwrcolka.cr.usgs.gov/uo/proposals/CRproposalweb.html) |  |  |  |  |  |  |  |
| NOTES: Space to provide additional information about program that might affect evaluation, but is not included in the proposal. |  |  |  |  |  |  |  |

SIGNATURES:																				DATE:

| Proposal Author: |  |
| --- | --- |
|  | Daniel Christiansen, Jon F. Nania, Jessica Garrett |
| Admin Officer Review: |  |
|  | Debra Malone |
| Center Approval: |  |
|  | Rob Middlemis-Brown |
| Region Approval: |  |
| Other: |  |

Proposal IA-09A

STREAMEST, a new approach for estimating stream flow and water quality

Summary

Surface-water flow information is the basis for understanding the hydrologic characteristics of streams and rivers. Water-quality information is of key importance to understanding the level of risk to human and ecosystem health within and beyond the watershed. Surface-water flow information in Iowa is collected on a nearly continuous basis at 128 streamgaging stations, and water-quality is collected on a monthly basis at 75 locations across Iowa.  However, streamflow and water-quality information is limited to these fixed locations despite an ongoing need for similar information at unmonitored locations throughout Iowa.  Limited period of record and analysis of observation data often does not account for changes in the stations’ hydrologic properties over time and implies stationarity of the observed data.  The stationarity within watersheds in Iowa change over time, evident in land-cover and land-use change, climate change, and many other environmental and human factors. Non-stationarity and the uncertainty in hydrologic properties do change over time, and having the capability to interpret the hydrologic information to account for the uncertainty is important in estimating hydrologic information (Milly and others, 2008). The ability to estimate surface-water flow and water quality at unmonitored sites in Iowa is critical for understanding hydrologic characteristics as well as estimating contaminant loading and flux and have been needed by State water managers. Current management techniques implemented by water managers are inadequate, and a new approach of adaptive management is required. Adaptive management allows for applying management practices as experiments, and to interactively assess, design, implement, monitor, evaluate, and adjust practices continuously (Murray, and others, 2003).

A comprehensive program that will develop predictive tools and models that are essential for use in understanding and managing Iowa waters for the future is needed. Such tools are needed to extrapolate or forecast conditions to unmonitored, yet comparable areas, both in space and in time. The combination of historic and continuously collected data is important in the overall development of these tools. Documenting the previously collected data is significant in developing and understanding networks, and is needed to improve upon them for future application development. Estimation methods that include statistical procedures and hydrologic modeling, and the comparison of these techniques to hydrologic events will aid in the development of the best applicable procedure in predicting streamflow and water-quality. Continual collection of streamflow and water-quality data will improve the parameters, site locations, and the capabilities of the estimation techniques.

A research-based comprehensive program is proposed to explore techniques and methods useful in developing near real-time estimates of streamflow and water-quality at unmonitored sites. Methods that produce the best results for Iowa will be selected and used to develop an interactive Web-based application that allows users to select stream locations for these estimates to be calculated. Estimates will include an uncertainty analysis with the goal of reporting a range of values rather than a specific value.

Results of the method evaluations and analysis will be documented through presentations and written reports.

Problem

The USGS (U.S. Geological Survey), in cooperation with State, county, municipal, and other Federal agencies, collects a large amount of data pertaining to the water resources of Iowa each year. These data, accumulated during many years, constitute a valuable data base for developing an improved understanding of the water resources of the State. Surface-water data for Iowa includes records of stage, discharge, and water quality of streams as well as stage of lakes and reservoirs. The present surface-water flow and stage gaging network consists of continuous records for 128 stations and peak-flow data for 90 crest-stage partial-record stations. Iowa has 71,000 miles of rivers and streams (IDNR, 2000), and the gaged sites on those streams only account for a very limited picture of the surface-water flow in the state.  The continuous records for the gaging stations are utilized for a wide range of hydrologic and environmental analyses including water quality and biological assessments (Table 1).  Similar to gaging networks, water-quality sampling networks have been established and funded by the Iowa Department of Natural Resources (IDNR), the U.S. Army Corps of Engineers, and the U.S. Geological Survey.  The sampling network consists of 84 IDNR and Urban sites, 10 USGS StreamEst (big rivers) sites, and 6 USGS nationally-funded sites (NAWQA and NASQAN programs) (figure #1, map of sites).

Excess nutrients and the presence of pesticides used in agricultural and urban areas have been documented in rivers and streams in Iowa. These contaminants can have deleterious effects on water quality in local Iowa streams, downstream on major rivers such as the Mississippi and Missouri Rivers, and potentially to the Gulf of Mexico. Until systematic state-wide monitoring of surface water quality was begun in 1999, sufficient data generally were not available to document the amount of nutrients and pesticides that were transported from the State.

With increasing attention focused on stream water-quality management, many water-resource regulatory agencies routinely require near real-time estimates of streamflow for many of the items listed in Table 1.  Near real-time estimation of streamflow at unmonitored sites historically has been calculated using methods that are not well documented, statistically unsupported, and do not have a known uncertainty analysis associated with the estimated value, such as using the drainage area ratio method (Eash, 2001).  There is a strong need by water-resource managers of the IDNR for a consistent and documented method for providing streamflow estimates at unmonitored sites in Iowa.  The estimate, at a minimum, is needed for both past and present conditions using the available surface-water network to calibrate the estimation.  There is a realization by State water-resource managers that the funding and operation of the streamgaging network would not be practical for expansion to all the sites that potentially could need streamflow information. An alternative is to develop statistical and modeling methods to be used as tools for streamflow estimation at unmonitored locations.  Similarly, funding and operation of water-quality sampling and monitoring networks is limited, and methods for extrapolating water-quality parameters to unmonitored locations and times are even less developed than streamflow estimation.  Water-quality measurements often cannot be made on a continuous or near real-time basis, but are usually collected as discrete data points, further limiting understanding of water quality, even at sites routinely monitored.

A continuation of consistent, representative water-quality data collection from the major tributaries and internal streams (figure #1) will allow the amount of nitrogen, phosphorus, and pesticides that are transported in Iowa rivers to be quantified. The continuation of data collection for a sufficient amount of time to determine how land-use changes and agricultural practices are changing the water quality in major rivers throughout Iowa is of key importance to the state agencies.

Therefore, there is a need to develop documented methods with quantifiable measures of uncertainty that can be used to estimate surface-water flow and water-quality at unmonitored sites on any stream in Iowa.  Ideally, the methods should provide an estimate of streamflow or concentration at any selected site in relation to networks, statistical, and modeling methods that have been developed. The physical and hydrologic similarity would be determined through comparison and significance of the watershed’s physical and environmental characteristics.

Table 1. Examples of Potential Uses of Estimated Surface-Water Flow Data

Environmental Studies

- Non-point source pollution

- Channel morphology evolution

- Sediment studies

- Wetlands ecology

- Tidal gate studies

- Vegetation studies

- Wildlife studies

- Fish studies

- Benthic studies

- Instream flow analysis

- Aquatic habitat studies

- Wild & Scenic determination

Hydraulic Design

- Roadways

- Bridges and culverts

- Dams, spillways, and reservoirs

- Channel modifications

- Flood-plain development

- Hydraulic modeling

- Urban beautification

- Navigable rivers for travel

Reservoir Management

- Routine operations

- Flood suppression

- Droughts

- Hydropower operation

- Scheduling bridge and dam inspections/repairs

Statistical Analysis

- Flood frequency

- Low flow frequency

- Flow duration

- Storage requirements

- Areal studies

- Safe yield analysis

Water Management

- Water supply, public and private

- Waste disposal allocation

- Water use

- Irrigation

- Emergency flood alert

- Water diversion permits

- Compliance with instream flow requirements

- Tide monitoring

Urban Studies

- Storm run-off

- Flood inundation

- Zoning and design regulations

- Pollution studies

- Scenic and wildlife suitability assessments

Water Quality

- Assimilative capacity

- Cumulative impacts assessment

- Baseline conditions

- Long-term trends

- Point-source and Non-point source impacts

- Interstate pollution transport

- Surface water – ground water relationships

- Salinity studies

- Dissolved oxygen studies

- Vegetation studies

- Nutrient loading studies

- Recreation suitability

- Regulatory monitoring

Recreation

- Canoeing activities

- Scenic river tour operations

- Sport fishing

- Competition rowing, swimming, waterskiing

- Pleasure boat

Objectives and Scope

The objective of this proposed program for (2009-2013) is to develop valid methods for estimating surface-water flow and water-quality at unmonitored sites. The subsequent supporting objectives will be completed with the main objective;

- Continuation of the stream-gaging network

- Streamflow estimation method development

- Modeled comparison of the floods of 1993 versus the floods of 2008

- Continuation of water-quality network information on major rivers in Iowa for model support, calibration, and verification

- 5 year report on major rivers water-quality

- Water-quality model of the Cedar River Watershed as an initial watershed for predictive water-quality analysis in Iowa.

- Collection of  ground-water climate response network information

The scope of the program is statewide.  The methods for estimation will have to account for physiographic, land-use, climatological, and watershed differences across Iowa.  Estimates of flow will be for mean daily streamflow and estimates of water-quality will be for concentration during the previous 24-48 hours; with the goal of reporting values to the public in the form of a range of values that includes the uncertainty of the estimate.  The actual mean daily estimated values of surface-water flow will be used internally with the IDNR for water-resource analysis and management.

Relevance and Benefits

This program supports the fundamental Water Resources Mission objectives of the USGS to provide reliable, impartial, and timely information about water resources (http://water.usgs.gov/mission.html).  Streamflow information is a core component of many of the Critical Water Resources Issues identified by the USGS Strategic Plan (http://pubs.usgs.gov/of/1999/ofr99-249/html/exec.html#H011) and the development of new techniques to estimate streamflow at unmonitored sites compliments the 5-year strategic directions identified for surface water.  Additionally, because the program will utilize Cooperative Water Program funds, the program meets the high-priority program objectives of the Cooperative Water Program’s 5-year plan (http://water.usgs.gov/usgs/prgmplans/coop_prgmplan.html) by increasing the availability of water-quality information, including real-time data, for rivers and coastal waters throughout the Nation and by enhancement of hydrologic-data networks through improved accessibility and presentation of available information and coordination of program activities with those of other agencies.

Estimation of streamflow and water-quality at unmonitored sites will assist State water-resource managers in making important decisions related to Iowa’s water resources.  In addition to aiding water-resource managers, the final products also can be used by the public for numerous uses.

Approach

The approach for this program is research-based.  Estimating flows and water-quality at unmonitored sites in near real-time has not been implemented extensively.  Various techniques and methodologies may be available and suitable to achieve the final objectives.  Figure 2 shows a flow chart of the objectives and approaches and the links between each. This program will explore these methods and determine those which produce the best results for this application.

Figure 2. Flow chart of program approach and development

Continuation of current data collection of streamflow, water-quality, and ground-water data across Iowa is a critical component for the Streamest program. Data from these networks will be used to calibrate models, improve statistical methods, and improve the accuracy of streamflow and water-quality estimates. Since these estimates will be based on the data from these networks, the data needs to be of the highest possible quality with minimal error.

The methods used to measure and compute stage and discharge values will be quality assured using nationally accepted protocols that have been extensively researched (Rantz and others, 1982). Discharge measurements will be routinely performed at stream-gaging sites during a variety of flow conditions to calibrate and verify stage-discharge relationships. The measurements will be made using highly sophisticated acoustic devices, and will follow strict protocols for quality assurance (Oberg and others, 2005).  The streamflow values calculated from these gages will be collected throughout the year so that flow can be computed at unmonitored locations at any point within the same timeframe. These streamgages will be located in various types of landforms, hydrologic regions, and watershed sizes in order to calculate streamflow representative of the variety of locations throughout Iowa.  Data, at these streamgages, will be collected at approximately 15-minute intervals to obtain the best possible data set to accurately compute mean daily streamflow at each streamgage.

Data from the streamgages will be transmitted in near real-time via satellite to the USGS downlink at which time flow calculations will immediately be performed and posted to the Internet.  This process is necessary because previous studies have shown trends in streamflow record. These trends can be the result of changes in climate, land use, or conservation practices (Schilling and Libra, 2003). The current data will be input into models and statistical equations to compute the flow at any selected unmonitored location. As conditions change, it is possible that data collected in the past may not pertain to current conditions. For this reason, long record periods still are required to test for trends in streamflow in Iowa. Near real-time transmission of data is critical because this will alert network managers of any malfunctioning equipment, which then can be quickly repaired as part of the on-going operation and maintenance needed at each streamgage.  Monitoring the data in near real-time will also provide the ability to obtain discharge measurements throughout a wide range of stages for updating and verifying the dynamic stage-discharge rating. Reliable and consistent estimates of streamflow at unmonitored locations within Iowa are only possible with a highly accurate network of streamgages.

The emergence of more sophisticated and accurate approaches to streamflow estimation through the use of statistical methods has occurred in recent years.  These methods are relatively new and are in need of additional research to validate their results.  Three statistical based methods will be examined for the StreamEst program.  All three of these methods will make use of the existing streamgage network in Iowa and streamgages within a 50-mile buffer in the surrounding states to estimate daily flow at unmonitored sites.  Current active gage stations will serve as index gages to the unmonitored locations.  The State will be divided into regions of similar hydrologic characteristics to ensure that flow is not computed from areas that are not hydrologically related to the unmonitored site, similar to the technique for developing the regions in Eash (2001).  To test the accuracy and compute an error for each of these methods, known gage locations will serve as unmonitored sites and estimated hydrographs will be created.  The estimated hydrographs will be compared with the observed hydrographs, and thus an estimate of error will be calculated for each method.

The first method will be an interpolation from nearby near real-time gages to the unmonitored location.  By using a combination of proximity and drainage area ratios to the unmonitored location, an index gage or gages will be selected that lies within the same hydrologic region.  After the index sites are selected, the unit discharge (cubic feet per square mile) for the desired day will be calculated and then multiplied by the drainage area of the unmonitored site to determine a discharge for that location.  If more than one index site is used, an average unit discharge will be used to compute the discharge at the unmonitored location.

A second method that will be tested is a flow duration-curve (FDC) transfer method.  This method estimates daily streamflow at an unmonitored location by equating the exceedence probability of a specific flow at one or more index or gaged locations with the exceedence probability at the unmonitored location (Vogel and Fennessey, 1994).  FDCs will be developed for all sites in the study area and a frequency distribution equation, most likely the Generalized Pareto or exponential distribution, will be computed for all sites.  Regional regression equations will be developed for the distribution equations based on the gaging network.  Using the regional distribution equations, an FDC can be created for an unmonitored site with the watershed characteristics specific to that location.  Once the FDC is developed for the unmonitored location, an average exceedence probability is calculated based on the index gages selected, and is assumed to be the same exceedence probability at the unmonitored location.  Based on this exceedence probability, a flow can be calculated for the unmonitored location from the computed FDC (Waldron and Archfield, 2006).

The third statistical-based method to be tested will be a regression-based approach.  Using the period-of-record daily discharge data at gaging stations in each hydrologic region, daily flows are regressed upon using significant watershed characteristics as explanatory variables.  This will result in a regression equation that can estimate flow at the unmonitored location based on the current conditions at index gages in the same hydrologic region.  This method will be able to compensate for time-series differences between an index site and the unmonitored site.

A comparison between the statistical approach and a Precipitation-Runoff Modeling System (PRMS) modeling approach will be included in the final report (Markstrom and others, 2008). The importance of comparing a physical based model with statistical methods will help understand the capabilities of both methodologies. The advantages and disadvantages of both methods will be documented.

Iowa had major flooding that occurred throughout the state in 1993, and in 2008 there has been a reoccurrence of major flooding throughout the state. In eastern Iowa, the Cedar River USGS streamgaging station at Cedar Rapids (05464500) had all time record streamflows and gage heights during June 2008. In 1993, the Cedar River hit a peak flow of 71,000 cubic feet per second (ft3/s), and a streamgage height of 19.27 feet (ft), in comparison to 2008, streamflow was 140,000 ft3/s, and 31.10 ft. The 2008 flood on the Cedar River was the largest historical flood of record at Cedar Rapids. The PRMS will be used to develop models of both the 1993 and 2008 major flood events along the Cedar River. The objective of the modeling is to compare and understand what caused the Cedar River floods of 1993 and 2008; the comparison will include similarities and differences of metrological, hydrological, landform, and land use conditions leading up to each event.

Water-quality samples will be collected using nationally accepted protocols documented in the USGS National Field Manual for the collection of water-quality data.  Constituents to be analyzed include field parameters (temperature, conductance, dissolved oxygen, pH, discharge), major ions (alkalinity, silica, chloride, sulfate), nutrients, algal pigments (chlorophyll a, pheophyton a), pesticides, sediment (suspended sediment, turbidity), and bacteria (E. coli, total coliforms). Sample sites and times will be selected to supplement or complement ongoing water-quality monitoring networks to provide both long-term data to evaluate trends, as well as stations intended for a limited period of record to address water-quality model calibration and validation needs.  Near real-time water-quality monitors will be installed and maintained at two streams for model calibration and verification.  These monitors also are intended for limited period of record (about 2 years) at sites which will change as model construction progresses.  Near real-time parameters include nitrate, temperature, conductance, and turbidity.

Consistent, representative water-quality data from the major rivers draining Iowa have been collected since 2004 (table 2).  With 5 years of the program completed, it is now time to document quantification of loads of nitrogen, phosphorus, sediment, and pesticides that are transported to the Missouri and Mississippi Rivers.  This knowledge will assist in the development of strategies to mitigate human impacts on the rivers and to evaluate the success (or failure) of these strategies once implemented, a critical step to the success of adaptive management goals.  The continuation of data collection for a sufficient amount of time to determine how land-use changes and agricultural practices are changing the water quality in major rivers throughout Iowa is of key importance to the state agencies.

### Table 2. Routine monitoring sites on major rivers draining Iowa

| Site ID | Site Name (USGS gage) | Drainage area (mi2) |
| --- | --- | --- |
| ## Iowa sites |  |  |
| 05412500 | Turkey River at Garber | 1,545 |
| 05418500 | Maquoketa River at Maquoketa | 1,553 |
| 05422000 | Wapsipinicon River nr DeWitt | 2,330 |
| 05474000 | Skunk River at Augusta | 4,303 |
| 05490500 | Des Moines River at Keosauqua | 14,038 |
| 06485500 | Big Sioux River at Akron | 8,424 |
| 06607500 | Little Sioux nr Turin | 3,526 |
| 06609500 | Boyer River at Logan | 871 |
| 06810000 | Nishnabotna River above Hamburg | 2,806 |
| 05465500 | Iowa River at Wapello | 12,499 |

The program includes a long term goal to develop the ability to estimate stream water quality along any stream at any point. The estimation methods need to include the ability to change watershed and hydrologic characteristics so as to simulate what water-quality changes would likely occur if the watershed characteristics were to change (for example if retention areas were constructed in the watershed or the impervious area was to increase). The Cedar River watershed will be modeled using the water-quality model: Soil & Water Assessment Tool (SWAT) (Winchell and others, 2007). The primary objective is to construct and calibrate a water-quality model for the Cedar River watershed, and develop methods within SWAT to estimate water-quality at any point along any stream in the Cedar River watershed. The estimates of water-quality will then be compared to observed data. A measure of accuracy will be developed between the model and observed data sets.

A ground-water climate response network has been established for nine climatological districts throughout Iowa to monitor shallow ground-water response to climatic conditions. The continuation of collecting ground-water data will support streamflow and water-quality modeling by providing data on base flow, soil-zone reservoirs, subsurface recharge, and ground-water recharge, interflow or subsurface flow from the nine climatological districts in Iowa. These nine wells will be equipped with telemetry that transmits a reading of water level by satellite. The data will typically be updated every 4 hours for Web-accessible viewing. These data constitute near real-time hydrologic data and will be reviewed or edited for publication in the annual data report. Continual collection of data at these sites will serve as support to both the streamflow and water-quality modeling by creating a historic record to create initial modeling parameters.

Throughout all phases of the program, on-going contact will be maintained with the IDNR to ensure that the program is meeting their needs.

Quality Assurance Plan

Achieving and maintaining high-quality standards for stream-gaging measurements is critical to achieving the goals of this program. In general, operation of gaging stations for the purpose of determining daily discharge includes the goal of collecting stage data at the accuracy of + or - 0.01 foot (Office of Surface Water memorandum 89.08). An explanation of USGS policy on stage-measurement accuracy as it relates to instrumentation is provided in Office of Surface Water memorandum 93.07. Quality assurance procedures for surface-water data collection are documented on the USGS Iowa Water Science Center’s internal Web page. http://ia.water.usgs.gov/usgs/reports/QAQCplan.book.pdf

The accuracy of streamflow measurements will have a direct impact on the water-quality data that is collected for this program.  High-quality representative water-quality data are required to minimize errors in estimation of constituent loads. The water-quality data objectives for the program are given in table 2. Field blanks and equipment blanks will be collected to ensure cross contamination does not occur due to contaminated equipment. Replicate samples will be collected to document analytical variability. Detailed quality-assurance procedures for water-quality data collection are documented on the Iowa Water Science Center’s internal Web page http://sw2diaiwc.cr.usgs.gov/usgs/nasqan/qa.pdf

The program includes written reports which will follow USGS report-writing procedures. Reports will go through the USGS and Iowa WSC report-review processes. The review process will include verification of manuscript with original data; a supervisor review with emphasis on technical accuracy, report organization, and completeness; colleague review to include specialists, and cooperators; an editorial review completed in the USGS publications network; Iowa WSC, and Region approval. Detailed information on report products and approval procedures can be found in Survey Manual 502.4.

Table 2.--Data-quality objectives for occurrence of constituents and calculation of loads.

[G, Gross detection/screening; SWQS, Surface-water-quality standards; AL, Aquatic-life protection; PDWS, Public drinking-water standards; W, whole water; D, dissolved; SS, Suspended sediment]

| Constituent group | Data-information objective | Phase or occurrence of constituent | Data quality objective (DQO) |
| --- | --- | --- | --- |
| Field parameters | G, SWQS, AL | W | IV |
| Major ions | G | D | IV |
| Nutrients | G, SWQS, PDWS, AL, load | D, W | IV |
| Algal pigments | G, SWQS, AL | W | IV |
| Pesticides | PDWS, AL, load | D | V |
| Suspended sediment | AL, load | W or SS | IV |
| Bacteria | G, CR | W | I |

DQO I – Gross detection at a point in the river

DQO IV – Detection (parts per million accuracy) in a representative cross section of the river

DQO V – Low-level detection (parts per billion accuracy) in a representative cross section of the river

Data collected from the existing streamgaging network of the Iowa WSC will be used to develop statistical methodologies used for estimating streamflow.  Iowa WSC staff will work closely with staff from the USGS Central Region, Headquarters, Office of Surface Water, and National Research Program to ensure that defensible scientific and statistical procedures and methods are implemented in the course of the program.

To test the relative success of each method for estimating near real-time flows at unmonitored sites, existing gages will be treated as unmonitored sites, and an attempt will be made to generate flows at that site with as little uncertainty as possible compared with the recorded flows.  The method that produces the results with the most confidence and the least amount of uncertainty will be considered the most appropriate.  To ensure that a method is appropriate for the entire range of recorded flow at a site, the entire period of record will be tested for each streamgage used in the program.

Surface-water and water-quality modeling for the development of estimation techniques is an interpretive process. The PRMS and SWAT models should appropriately represent the physical, chemical, and biological processes occurring with the watershed so that the runoff volume, sediment yield, and nutrient loads can be accurately qualified. Model calibration reduces the uncertainty and errors related to model predictions. Calibration of the PRMS and SWAT models for both flow and nutrients is assessed using statistical measures (Nash-Sutcliffe Efficiency, Root Mean Square Error, and Coefficient of Determination) as well as visual comparisons (observed vs. predicted and time series graphs). The model calibration will appropriately represent the physical characteristics and process within the watershed.
 	The Iowa WSC has an established set of guidelines and practices to be used to assure quality in ground-water activities.  Procedures for quality assurance in the Iowa Water Science Center ground-water quality assurance plan generally conform to guidelines presented in the National Quality-Assurance Plan for Center Ground-Water Activities (Brunett and others, 1997; Appendix A). Ground-water quality assurance procedures for data collection are documented on the Iowa Water Science Center’s internal Web page.     http://ia.water.usgs.gov/usgs/reports/qapgw.pdf

All computer files (QW data, calculation of constituent loads, digital photos, etc) will be maintained in project directories on the Water Science Center’s Server to ensure proper backup and ensure access by all project personnel. Because the program encompasses the entire state, any new spatial data layers (GIS coverages) will be documented and saved in the Water Science Center State and Region directories. A project GIS directory will be created on the Water Science Center Server for interim and specialized project coverages.

Products

The results of the research being conducted in this program will include data collection, modeling, statistical prediction methods, reports, and real-time data.  The products for each objective of the program will be unique but will be part of the program’s overall goal of stream and water quality estimation techniques.

Stream-gaging is an ongoing data collection product of this program. Data will be continuously collected in near real-time, and transmitted to the USGS via satellite and posted to the internet. Stream-gage data will be compiled into an Annual Data Report and published for each year of data collection.

Water-quality sampling will be collected at selected sampling sites for both long-term trends, and short-term sites for model input needs. Near real-time water-quality monitors will be installed and continuously collect and upload via satellite to the USGS and internet. A Scientific Investigations Report is to be completed for the first 5 years of the water-quality long term sites and to address loadings to both the Mississippi and Missouri Rivers.

Streamflow estimation techniques using statistical and modeling methods are being developed. Three statistical methods are being examined and two PRMS models are being constructed to estimate streamflow. A Scientific Investigations Report will be written documenting the statistical and modeling techniques that have been tested. A web-interface will be developed for the IDNR that will implement the techniques tested to estimate flows on unmonitored streams in Iowa.

PRMS models of the major floods of 1993 and 2008 will be constructed. An analysis of both floods will be conducted to examine the environmental conditions before, during, and after the floods, and how environment conditions compare between both major flood events. The results of the analysis will be documented in a report.

Water-quality modeling will be used to develop techniques for estimating water-quality at unmonitored sites. A water-quality model will be constructed for the Cedar River watershed as a test watershed for estimating parameters. Development of water-quality models in other watersheds that are to be determined will be constructed. Water-quality models will incorporate both the streamgage, and water-quality network data for calibration and verification purposes. A Scientific Investigations Report will be written documenting the water-quality models, and their relevance to the program objectives.

Ground-water climate response network data will be collected continuously at nine climatological locations. The ground-water data will be used in support of streamflow and water-quality modeling efforts. Data will be continuously collected and transmitted to the USGS via satellite and posted to the internet in near real-time. The ground-water data will be published with the Annual Data Report.

Work Plan for FY2009-13

|  |  | 2009 |  |  |  | 2010 |  |  |  | 2011 |  |  |  | 2012 |  |  |  | 2013 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| Continue SW Network |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Data Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Continue QW Network |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5-year report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Annual Data Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Streamflow Estimation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Statistical Methods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRMS modeling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interpretive Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Web Interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Quality Estimation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SWAT model Cedar River |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Interpretive Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Web Interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GW Climate Network |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Collection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Report |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Personnel

Personnel for this program are currently available within the Iowa Water Science Center.

Budget

The following costs (for USGS only) include all benefits and indirect costs.

|  | Cost of study activities, in dollars per year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Project Activity |  | 2009 | 2010 | 2011 | 2012 | 2013 |
| Continue SW Network |  |  |  |  |  |  |
| Data Collection |  | $360,060.00 | $372,662.00 | $385,705.00 | $399,204.00 | $413,177.00 |
| Annual Data Report |  | $10,000.00 | $10,350.00 | $10,712.00 | $11,087.00 | $11,475.00 |
| Continue QW Network |  |  |  |  |  |  |
| Data Collection |  | $320,800.00 | $332,028.00 | $343,648.00 | $355,676.00 | $368,125.00 |
| 5-year report |  | $68,500.00 | $35,448.00 |  |  |  |
| Annual Data Report |  | $5,000.00 | $5,175.00 | $5,356.00 | $5,543.00 | $5,737.00 |
| Streamflow Estimation |  |  |  |  |  |  |
| Statistical Methods |  | $37,300.00 |  |  |  |  |
| PRMS modeling |  | $80,400.00 | $62,410.00 |  |  |  |
| Interpretive Report |  |  | $70,897.00 | $36,689.00 |  |  |
| Web Interface |  |  |  | $50,000.00 | $103,500.00 | $51,750.00 |
| Water Quality Estimation |  |  |  |  |  |  |
| SWAT model Cedar River |  | $72,100.00 | $74,623.00 |  |  |  |
| Interpretive Report |  |  |  | $73,378.00 | $56,960.00 |  |
| Web Interface |  |  |  |  | $75,000.00 | $77,625.00 |
| GW Climate Network |  |  |  |  |  |  |
| Data Collection |  | $35,840.00 | $37,094.00 | $38,392.00 | $39,736.00 | $41,127.00 |
| Annual Data Report |  | $5,000.00 | $5,175.00 | $5,356.00 | $5,543.00 | $5,737.00 |
| TOTAL |  | $995,000.00 | $1,005,864.00 | $949,239.00 | $1,052,252.00 | $974,755.00 |

Safety

See attached Job Hazard Analysis; all field and office operations will follow and adhere to established and approved safety guidelines and protocol.

Archival Plan

Program financial and management files, correspondence, and appropriate technical or data analyses will be archived in the office files of the Iowa Water Science Center according to procedures already in place.  These procedures will be document and on file in the future. Iowa Water Science Center data management plan can be found at (http://ia.water.usgs.gov/usgs/data/IADataManagementPlan.doc)

References

Brunett, J.O., Barber, N.L., Burns, A.W., Fogelman, R.P., Gillies, D.C., Lidwin, R.A., and Mack, T.J., 1997, A quality-assurance plan for Center ground-water activities of the U.S. Geological Survey: U.S. Geological Survey Open-File Report 97-11, 21 p.

Eash, D.A., 2001, Techniques for estimating flood-frequency discharges for streams in Iowa: U.S. Geological Survey Water-Resources Investigations Report 00-4233, 88 p.

IDNR (Iowa Department of Natural Resources), 2000, Iowa water monitoring plan, 12 p.

Maidment, D.R., 1993, Handbook of hydrology:  Frequency analysis of extreme events, McGraw-Hill, Inc., p. 18-1 to 18-66.

Markstrom, S.L., Niswonger, R.G., Regan, R.S., Prudic, D.E., and Barlow, P.M., 2008, GSFLOW—Coupled ground-water and surface-water flow model based on the integration of the Precipitation-Runoff Modeling System (PRMS) and the Modular Ground-Water Flow Model (MODFLOW-2005): U.S. Geological Survey Techniques and Methods 6-D1, 240 p.

Milly, P.C.D., Betancourt, J., Falkenmark, M., Hirsh, R.M., Kundzewicz, Z.W., Lettnmaier, D.P., Stouffer, R.J., 2008, Stationarity Is Dead: Whither Water Management?, Science, V. 319, no. 5863, pp. 573-574.

Murray, C. and D. Marmorek. 2003. Adaptive Management and Ecological Restoration. Chapter 24, in: Freiderici, P. (ed.). 2003. Ecological restoration of Southwestern Ponderosa Pine Forests. Island Press (Washington, Covelo CA, London), pp.417-428

Oberg, K.A., Morlock, S.E., and Caldwell, W.S., 2005, Quality-assurance plan for discharge measurements using acoustic Doppler current profilers, U.S. Geological Survey Scientific Investigations Report 2005-5183, 35 p.

Rantz, S.E., and others, 1982, Measurement and computation of streamflow, Volumes 1 and 2, Measurement of discharge:  U.S. Geological Survey Water-Supply Paper 2175, 631 p.

Schilling, K.E. and Libra, R.D., 2003, Increased baseflow in Iowa over the second half of the 20th century: Journal of the American Water Resources Association, vol. 39, p. 851-860.

Vogel, R.M., and Fennessey, N.M., 1994, Flow duration curves I: new interpretation and confidence intervals: Journal of Water Resources Planning and Management, V. 120, no. 4, p.485-504.

Waldron, M.C., and Archfield, S.A., 2006, Factors affecting firm yield and the estimation of firm yield for selected streamflow-dominated drinking-water supply reservoirs in Massachusetts: U.S. Geological Survey Scientific Investigations Report 2006-5044, 39 p.

Winchell, M., R. Srinivasan, M. Di Luzio, and J. G. Arnold (2007), Arc-SWAT interface for SWAT2005 - User’s guide, Blackland Research Center,Texas Agricultural Experiment Station and Grassland, Soil and Water Research Laboratory, USDA Agricultural Research Service, Temple, Texas

| Proposal Job Hazard Analysis – Central Region  Check the numbered box(s) for all significant safety concerns this program should address. Significant safety concerns are commonly those that require training, purchase of safety equipment, or specialized preparation to address potentially hazardous conditions.  Identify any unlisted safety concerns at bottom of the page.  Provide details on the back of this page. Proposal Number IA-09A Program Title (Short) SW Estimation Program Chief or Proposal Author Daniel Christiansen |  |
| --- | --- |
|  | # Potential Program Safety Elements |
| 1.  | Wading, bridge, or cableway measurements or sampling (WRD 99.32 & 01.05) |
| 2.  | Working on ice covered rivers or lakes (see WRD 00.03) |
| 3.  | Measuring or sampling during floods |
| 4. | Well drilling; coring, augers, hydro-punch, borehole logging |
| 5.  | Electrical hazards in the work area – above and below ground |
| 6. | Construction – including cableways, trenching and demolition |
| 7.  | Working in remote areas, communication, office call in procedures (OP94.02) |
| 8.  | Ergonomics, Office issues, carpal tunnel syndrome |
| 9.  | Field Vehicles appropriate for task? - Safety screens equipment restraints. |
| 10.  | All terrain vehicles, snowmobiles, fork lifts, |
| 11. | Helicopter or fixed wing aircraft usage (see OAS at: http://www.oas.gov/ ) |
| 12.  | Site access: Federal, State, County and private lands |
| 13.  | Hypothermia or Hyperthermia (heat stress) |
| 14.  | Hantavirus, Lyme Disease, Histoplasmosis, Pfiesteria, Others? |
| 15.  | Contaminated water or soil with sanitary, biological, or chemical concerns |
| 16.  | Immunizations - voluntary programs |
| 17.  | Laboratory or mobile laboratory. Chemical hygiene plan, HazComm & MSDS’s |
| 18.  | Hazardous waste disposal – Lab and Field |
| 19. | Hazardous waste site operations (RCRA, CERCLA) HASP, HAZWOPPER |
| 20.  | Confined space – Stilling Wells, Well Pits, Sample sites |
| 21. | Radioactivity – Borehole logging – Soil Moisture - |
| 22. | Respiratory protection – Dusts, Vapors, Fumes, Biologic (medical monitoring) |
| 23.  | Water levels – wells, well pits, pumps and electrical issues |
| 24.  | Electrofishing (see http://1stop.usgs.gov/safety/Topic/jha/electrofishing.htm ) |
| 25.  | High pressure compressed gas cylinders – handling and transport |
| 26.  | Boating – operator training, equipment, requirements, inspections |
| 27. | Special Training |
| 28. |  |

Discussed job hazard analysis (JHA) with Center Collateral Duty Safety Officer and/or copy of JHA given to Collateral Duty Safety Officer     Yes____ No__X__

Center Director __________________________________________

Regional Safety Officer __________________________________Date ___________