PROPOSAL

web-based streamflow statistics tool for nebraska (streamstats)

by

Richard C. Wilson, P.E. and BenjamiN Dietsch

U.S. Geological Survey (USGS)

Nebraska WateR Science Center (NEWSC)

December 12, 2006

PROBLEM and Background

Information about streamflow characteristics of Nebraska’s rivers and streams are needed by government agencies, private companies, and the general public for multiple purposes. Estimates of the magnitude and frequency of flood-peak discharges and flood hydrographs are needed for a variety of purposes, such as the design of bridges, culverts, and flood-control structures, and for the management and regulation of flood plains. Estimates of low flow and drought flows are needed for water resources planning, determination of conditions for pollutant-discharge permits, calculations of Total Daily Maximum Loads (TMDLs), and other ecological impact studies.

At streamflow data collection stations (gages) where sufficient long-term streamflow data have been collected, discharges and selected statistics can be obtained from the USGS National Water Information System (NWIS) database. However, estimates are needed more often at ungaged sites where no observed streamflow data are available.  Regression equations that statistically relate streamflow statistics to basin characteristics, such as drainage area, topography, soil type, and precipitation, can be used to predict discharges at ungaged sites.  The drainage-basin characteristics are measured for an ungaged site and used as input into the regression equations to obtain estimates for the ungaged site.

Historically, manual methods have been used to compute delineate the drainage basin and measure basin characteristics for ungaged sites; however, these manual techniques are slow, resource intensive, and may produce non-reproducible results. Often manual methods are not feasible for the public and non-hydrologists because they require training, multiple data sources, and expensive computer software and resources. Geographic Information System (GIS) procedures using digital datasets have improved the estimation of basin characteristics, however, without adequate datasets, software, and training, GIS techniques of defining basin characteristics may be inaccurate and could potentially produce incorrect streamflow statistics.

OBJECTIVES AND SCOPE

The objective of this study is to provide a web-based user interactive tool (StreamStats) that can deliver existing streamflow data and statistics for gaged sites and that can estimate selected peak- and low-flow statistics at ungaged sites in Nebraska. To achieve the objective, it will be necessary to develop new regression equations. Currently the USGS does not have low flow equations for NE and the equations for peak flow will be updated with improved accuracy.

StreamStats will require the user to click on a map to identify the stream location of interest; the program will then: delineate the drainage basin for the selected location, determine the required basin characteristics, and calculate streamflow statistics from the available regression equations. It will provide a downloadable set of results that will include a map of the watershed and a streamflow statistical summary that includes confidence intervals and an explanation of the results. StreamStats will be available to government agencies, private companies and the general public so that information can be quickly obtained. StreamStats will be a locally applied component of a nationally standardized analytical tool that is being developed by the USGS Office of Surface Water.

RELEVANCE AND BENEFITS

This proposed U.S. Geological Survey (USGS) Cooperative Water Program (CWP) study will be implemented in cooperation with the Nebraska Department of Roads and the Nebraska Department of Environmental Quality. The results of the study will provide StreamStats to government agencies, private companies, and the general public. This project specifically fulfills the mission of the USGS to focus on responsiveness and service to customers. It provides commonly needed, scientifically defensible, uniform and non-biased hydrologic information to the Nebraska hydrologic community. This project will advance hydrologic research and thereby fulfill this science–related mission. It is a predictive tool that will provide flood magnitude and frequency estimates, as well as low-flow estimates of discharge. This project benefits the Nebraska Water Science Center (NEWSC) by focusing on priority science issues and achieving our water resources mission.

APPROACH

The product will be a Nebraska version of the nationally developed StreamStats application by the USGS Office of Surface Water. The approach and methodology for this application is outlined on the StreamStats web page (http://water.usgs.gov/osw/programs/streamstats.html).

StreamStats will provide a user interface that presents data for both gaged and ungaged sites. For existing long-term gaged sites, data are provided for previously published historical streamflow statistics and basin characteristics. For ungaged streamflow sites, the basin characteristics needed for regression parameters are calculated by using GIS datasets, and then regression equations estimate peak- or low-streamflow statistics. The following work elements will be accomplished for this project:

- Project Management  Manage budget, project reviews & updates

- Research & identify basin characteristics  Past regional analyses for Nebraska and for surrounding states will be reviewed to determine basin characteristics that have been used in those studies to explain regional variation in streamflow statistics. A group of basin characteristics will be selected for measurement and measurement methods will be determined.

- Preparation of GIS Datasets for the Study Area.  GIS datasets for delineating drainage-basin boundaries, for measuring basin characteristics, and for locating sites of interest will need to be prepared. For drainage-basin boundary delineation, a digital elevation model (DEM) dataset will be forced to agree with previously digitized hydrography and watershed boundaries datasets to increase the accuracy of the delineations.  As a result, delineations obtained from StreamStats usually will be of greater accuracy than delineations obtained from standard non-drainage enforced DEMs.  The conditioned DEM and any other datasets needed for measuring basin characteristics will be structured for use within the ArcHydro framework (Maidment, 2002), which will allow for use of the datasets for a multitude of purposes in other water-resources projects. GIS datasets needed locating sites of interest, such as streamgage locations and digital topographic map images, will also be prepared.  All new datasets will have Federal Geographic Data Committee (FGDC) compliant metadata (Federal Geographic Data Committee, 1994).

- Statistics at Continuous-Record Gages  The current and historical stream-gaging network for Nebraska will be reviewed to select station appropriate for use in regression analysis.  Some stations close to Nebraska in adjacent states having physical and hydrologic properties similar to Nebraska stations will likely also be selected. Data from the NWIS will be downloaded and formatted for use in statistical software that will be used to determine the streamflow statistics.  Kendall's tau tests will be used to check for trends in annual time series.  Stations with trends will either be eliminated from further study or the time series will be adjusted to eliminate the trends.  Peak-flow and low-flow frequency statistics will be computed for the stations, the fit of the frequency curves will be assessed, and adjustments will be made, if necessary.  Flow-duration and other selected streamflow statistics will also be computed, and all computed statistics will be compiled into a database for regression analysis,

- Measurement of basin characteristics at continuous-record gages  The ArcHydro Toolset will be used to measure all basin characteristics for use in the regression analysis. Use of the Toolset assures that the measurement methods will be the same as those used in StreamStats, thus differences in the bias and accuracy of the measurements will be avoided.

- Regression Analyses  Regression equations will be developed that can be used to estimate streamflow statistics for peak- and low-flow conditions on ungaged streams throughout Nebraska. The equations will be developed by regression analysis to statistically relate streamflow characteristics to basin characteristics for similar drainage basins.  Logistic, weighted-least-squares (WLS), and generalized-least-squares (GLS) regression procedures will be used.  An initial analysis using logistic and WLS regression will be done using all stations within the study region.  Logistic regression will be used to develop a preliminary equation for determining the probability of zero flow. WLS will be used to develop preliminary equations for other streamflow statistics, with weighting based on the length of record for each station.  Differences between the observed streamflow statistics and those predicted by the state-wide equations, and other information, will be used to develop hydrologic regions within Nebraska.  Separate regression equations will be developed for each identified hydrologic region.  The peak-flow frequency equations will be developed using GLS regression, which weights the stations based on cross-correlation of the annual peak-flow time series as well as on record length.

The regression equations will apply to basins having little or no regulation and where significant urban development has not occurred. StreamStats extensions to account for heavily regulated and urbanized are still in development. Therefore, the initial phases of the project will focus on the unregulated drainage basins and the later phase will address regulated basins.

- Drainage-Area Ratio Method  In addition to use of regression equations for estimating streamflow statistics at ungaged sites, StreamStats will also be able estimate the statistics based on the drainage-area ratio method of applying the flow per unit area for an upstream or downstream gaging station to the ungaged site.  By default, this method will only be applicable if the drainage area for the ungaged site is within 0.5 and 1.5 times the drainage area for the gaging station.  

Applicability of the drainage-area ratio method will be tested for Nebraska by identify a group of unregulated river basin with multiple gaging stations.  For each station, streamflow statistics will be computed from the available record at the station, and estimated using regression equations and the drainage-area ratio method.  Comparisons between the computed and estimated statistics will be done to determine the limits of the drainage-area ratio within which that method generally provides more accurate estimates than the regression equations.

- StreamStats Setup, Operation, Testing and Documentation. The GIS datasets, regression equations, and other materials needed to implement StreamStats will be delivered to the national implementation team, which will set up the application.  The major component of the StreamStats user interface consists of the Map Frame (figure 1), which displays default and selected map layers. Within the Map Frame, map layers can be changed and the user can query information on selected map layers or features, delineate drainage boundaries, and other hydrologic information from the NWIS.  

The user interface will allow the user to navigate the digital map and select gaged and ungaged sites.  When a gaged site is selected, the user will be provided with previously published streamflow statistics and basin characteristics information from a database. When an ungaged site is selected, StreamStats will determine what regression equations are available for the ungaged site and what basin characteristics are needed to solve the equations.  It will then measure the basin characteristics and solve the regression equations to estimate various streamflow statistics.  Prior to public release, testing by the USGS and our cooperators will be conducted to ensure the system is operational and serving data and estimating flow correctly.

- Scientific Investigation Report and Fact Sheet  The study will produce a USGS Fact Sheet to inform users and the public of the availability and use of StreamStats and a Scientific Investigations Report (SIR) to document the development of the peak- and low-flow regression equations, streamflow statistics and basin characteristics.

- Streamflow Data and Statistics at Gaged Sites.  After approval of the SIR, Streamflow data will be assembled for the StreamStats database (StreamStats DB) that will consist of published streamflow statistics, descriptive information, and basin characteristics for USGS gages. Streamflow statistics will include: peak- and low-flow frequencies, annual and monthly means, medians, and flow-duration percentiles.  Descriptive information in StreamStats DB includes: USGS station identification, station name, station type, period of record, latitude, longitude, hydrologic unit code, major drainage basin name, county name, directions to the gage, and other pertinent information. This will provide convenient access for users to previously published data through the StreamStats interface. Specific tasks include:

Figure 1.  Graphic showing National StreamStats web site prototype, A. Delineating a watershed for the selected site. B, Displaying streamflow statistics estimated for selected site.

A.

B.

Quality Assurance Plan.   All products and deliverables generated by this study will be checked for completeness and accuracy and quality assured by hydrologists in the NEWSC and by other qualified professionals according to the peer review requirements of the USGS.  The generated regression equations will be reviewed and tested against available peak- and low-flow discharge measurements to ensure their functionality and accuracy.  All data will be quality assured according to standard USGS procedures and archived in the USGS National Water Information System (NWIS) database, where it will be available to the public. GIS generated watershed parameters at existing gages will be checked against historical data in the NWIS and by independent verification using manual methods. StreamStats statistics produced will be verified using historical data from existing gaging stations.  New GIS datasets produced as part of this project will be reviewed by using digital raster graphics (DRG) maps as ground-truth information; the target vector-accuracy standard will be 1:25 inch at the 1:24,000 scale. All new datasets will have Federal Geographic Data Committee (FGDC) compliant metadata (Federal Geographic Data Committee, 1994).

PRODUCTS

The products produced by this study will be published to ensure that the information is disseminated to government agencies, private companies, and the general public.  Three primary products will include:

- The peak- and low-flow frequency analysis for the NEWSC streamflow gaging network and the resulting peak- and low-flow relationships for ungaged streams will be published as a USGS Scientific Investigations Report (SIR).

- The Nebraska StreamStats interactive web-based user tool will be the primary product of this project.  A USGS Fact Sheet (in hardcopy and digital format) will be published to summarize this web-based tool.

- GIS hydrologic datasets for the study area (elevation, hydrography, watershed boundaries, and site locations) will be published digitally.

WORK PLAN

| Workplan Element | FY07 |  | FY08 |  |  |  | FY09 |  |  |  | FY10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Apr-Jun | Jul-Sep | Oct-Dec | Jan-Mar | Apr-Jun | Jul-Sep | Oct-Dec | Jan-Mar | Apr-Jun | Jul-Sep | Oct-Dec | Jan-Mar |  |
| (I) Project Management | x | x | x |  | x | x | x | x | x | x | x | x |  |
| (II) StreamStats Setup | x | x | x | x |  |  |  |  |  |  | x | x |  |
| (III) Statistics at Gages |  |  | x | x | x |  |  |  |  |  |  |  |  |
| IV) Regression Analyses |  |  |  |  |  | x | x | x | x |  |  |  |  |
| (V) Drainage-Area Ratio Method |  |  |  |  |  | x | x | x | x |  |  |  |  |
| (VI) Scientific Investigation Report and Fact Sheet |  |  |  |  |  |  |  |  |  | x | x | x |  |

Budget

|  | FY07 | FY08 | FY09 | FY10 | Total |
| --- | --- | --- | --- | --- | --- |
| Cooperator | $143,500 | $289,500 | $171,600 | $90,400 | $695,000 |
| USGS | $25,300 | $51,100 | $30,300 | $16,000 | $122,700 |
| Total | $168,800 | $340,600 | $201,900 | $106,400 | $817,700 |

REFERENCES

Federal Geographic Data Committee. 1994. Content standards for digital spatial metadata (June 8 draft). Federal Geographic Data Committee. Washington, D.C.

Maidment, David, ed., 2002, Arc Hydro: GIS for Water Resources: ESRI Press, Redlands, California.

Ries, Kernell G., ed., 2004, StreamStats: A U.S. Geological Survey Web Application for Stream Information, USGS Fact Sheet FS 2004-3115.

Soenksen, Philip J., ed., 1999, Peak-Flow Frequency Relations and Evaluation of the Peak-Flow Gaging Network in Nebraska; USGS Water-Resources Investigations Report 99-4032.