| Turbidity parameter and method codes** |  |  |  |
| --- | --- | --- | --- |
| Method long name | Method code | pcode | Parameter long name |
| D-A Instruments Co, sensor model OBS-3A, FBU | TS002 | 63682 | Turbidity, water, unfiltered, monochrome near infra-red LED light source, 780-900 nm, detection angle 0 degrees to incident light (backscatter), FBU |
| Eureka Environmental, sensor model Trimeter, FNU | TS031 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| Forest Technology Systems, sensor model DTS-12, FNU | TS032 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| HACH, sensor model 2100 AN (Ratio OFF), NTU | TS097 | 63675 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detection angle 90 +/- 30 degrees to incident light, NTU |
| HACH, sensor model 2100 N (Ratio OFF), NTU | TS099 | 63675 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detection angle 90 +/- 30 degrees to incident light, NTU |
| HACH, sensor model 2100 P, NTRU | TS101 | 63676 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detectors at multiple angles including 90 +/- 30 degrees, ratiometric correction, NTRU |
| HF Scientific, sensor model Micro100 (light source WHITE), NTU | TS108 | 63675 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detection angle 90 +/- 30 degrees to incident light, NTU |
| Hydrolab, Sensor model 4 Beam, FNMU | TS011 | 63684 | Turbidity, water, unfiltered, monochrome near infra-red LED light source, 780-900 nm, multiple beam, detectors at multiple angles including 90 degrees, FNMU |
| Hydrolab, sensor model Datasonde 4, FNU | TS054 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| Hydrolab, Sensor Model Datasonde 4a, FNU | TS053 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| Hydrolab, sensor model Quanta (4-beam), FNMU | TS012 | 63684 | Turbidity, water, unfiltered, monochrome near infra-red LED light source, 780-900 nm, multiple beam, detectors at multiple angles including 90 degrees, FNMU |
| In-Situ, sensor model MP TROLL 9000, FNU | TS055 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| LaMotte Instruments, sensor model 2008, NTU | TS114 | 63675 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detection angle 90 +/- 30 degrees to incident light, NTU |
| McVan, sensor model Analite NEP 195, FNU | TS060 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| McVan, sensor model Analite NEP 395, FNU | TS063 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| Thermo Electron Corp., Sensor Model, Orion AQUAFast AQ4500, (Light-source INFRA RED Attenuation ON), FAU | TS029 | 63683 | Turbidity, water, unfiltered, monochrome near infra-red LED light source, 780-900 nm, detection angle 180 degrees to incident light (attenuation), FAU |
| Thermo Electron Corp., Sensor Model, Orion AQUAFast AQ4500, (Light-source WHITE, Attenuation ON), AU | TS072 | 63678 | Attenuation turbidity, water, unfiltered, broad band light source (400-680 nm), detection angle 180 degrees to incident light, AU |
| Thermo Electron Corp., Orion AQUAFast AQ4500, (Light-source INFRA RED, Ratio ON), FNRU | TS073 | 63681 | Turbidity, water, unfiltered, monochrome near infra-red LED light, 780-900 nm, detectors at multiple angles including 90 +/- 2.5 degrees, ratiometric correction, FNRU |
| Thermo Electron Corp., Sensor Model Orion AQUAFast AQ4500, (Light source INFRA RED, Ratio OFF), FNU | TS074 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| Thermo Electron Corp., Sensor Model, Orion AQUAFast AQ4500, (Light-source WHITE, Ratio ON), NTRU | TS123 | 63676 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detectors at multiple angles including 90 +/- 30 degrees, ratiometric correction, NTRU |

| WTW Measurement Systems, sensor model 350 IR, FNU | TS081 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| --- | --- | --- | --- |
| WTW Measurement Systems, sensor model 550 IR, FNU | TS082 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| WTW Measurement Systems, sensor model 555 IR, FNRU | TS083 | 63681 | Turbidity, water, unfiltered, monochrome near infra-red LED light, 780-900 nm, detectors at multiple angles including 90 +/- 2.5 degrees, ratiometric correction, FNRU |
| WTW Measurement Systems, sensor model 555, NTRU | TS129 | 63676 | Turbidity, water, unfiltered, broad band light source (400-680 nm), detectors at multiple angles including 90 +/- 30 degrees, ratiometric correction, NTRU |
| WTW Measurement Systems, sensor model VisoTurb 700 IQ, FNU | TS084 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| YSI Environmental, sensor model 6026, FNU | TS085 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| YSI Environmental, sensor model 6026-4000, FNU | TS086 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |
| YSI Environmental, sensor model 6136, FNU | TS087 | 63680 | Turbidity, water, monochrome near infra-red LED light, 780-900 nm, detection angle 90 +/ -2.5 degrees, FNU |

** This is a list of commonly used turbidity instruments.  If your instrument isn’t on this list, see the complete list of turbidity codes at http://water.usgs.gov/owq/FieldManual/Chapter6/6.7_contents.html